RDK-B Raspberry Pi 4
Access Point Setup Guide

A Comprehensive Walkthrough for Platform Build, Package Integration, Architecture, and AP Configuration

Reference Platform: Raspberry Pi 4 Model B
Environment: Yocto Project (Kirkstone Branch)

Guide Overview

This structural guide systematically walks through configuring a Raspberry Pi 4 as an operational Access Point (AP) running the Reference Design Kit for Broadband (RDK-B) ecosystem. The configuration path includes ecosystem breakdown, host workspace initialization, compilation flags, package payload changes, physical flashing, runtime execution, and troubleshooting steps.

1. RDK-B Architecture Breakdown

2. Build Host Machine Requirements

3. Cloning Yocto Source Repositories

4. Initializing Build Environment

5. Configuring Raspberry Pi 4 Target

6. Adding Required Meta-Layers

7. Adding Access Point Packages Before Build

8. Executing the Yocto Image Build

9. Verifying the Generated Target Image

10. Decompressing the Image File

11. Inserting and Detecting the SD Card

12. Unmounting Local SD Card Partitions

13. Flashing the Target Image to SD Media

14. Safe Hardware Removal

15. Booting the Raspberry Pi Hardware

16. Verifying Installed AP Packages

17. Confirming Wireless Interface State

18. Configuring WLAN Interface IP Layer

19. Creating hostapd System Configuration

20. Creating dnsmasq DHCP Configuration

21. Launching hostapd Daemon

22. Starting dnsmasq Infrastructure Daemon

23. Verifying Access Point Runtime State

24. Connecting Client Devices

25. Monitoring Connected Client Stations

26. Validating Lease & DHCP Assignment

27. Comprehensive Troubleshooting Guide

1. RDK-B Architecture

RDK-B (Reference Design Kit - Broadband) is a highly integrated, open-source software platform explicitly designed for broadband gateways, routers, and high-performance wireless access points. It builds a standardized layer above hardware specific architectures to provide predictable behavioral stacks.

Platform Functional Architecture Layers

Web UI / Remote Management Layer

CCSP Components (WiFi ¦ DHCP ¦ Firewall ¦ Routing ¦ Telemetry)

RDK-B Middleware Stack

Standard Linux Networking Stack

Wireless Drivers (Vendor Specific / Open Source)

Linux Kernel Space

Raspberry Pi 4 Hardware Base

Wireless Subsystem Stack Architecture Flow

The interaction between user-space control utilities and low-level physical kernel-space radios moves through standardized programmatic entry-points:

User Space Components:

hostapd (Access Point Daemon Handling Auth/Association) dnsmasq (Lightweight DHCP Lease and DNS Allocator) iw (Modern nl80211-based Wireless CLI Engine) wpa_supplicant (Client Security Framework)

Communication Bridge Protocol:

nl80211 Netlink Interface

Kernel Space Subsystems:

cfg80211 (Linux Wireless Configuration API) mac80211 (Media Access Control Soft-MAC Layer Device Driver Core) WiFi Physical Driver Layer --> Physical Broadcom WiFi Hardware Interface

2. Build Host Machine Requirements

The Yocto/Poky engine demands a robust Linux compilation workstation environment. It is strongly recommended to use a native Ubuntu 20.04 LTS or 22.04 LTS installation.

Execute the following terminal routine to synchronize system repositories and provision all required structural compilation toolchains, host packages, and dependency parsing tools:

sudo apt update sudo apt install gawk wget git-core diffstat unzip texinfo gcc build-essential \ chrpath socat cpio python3 python3-pip python3-pexpect xz-utils \ debianutils iputils-ping libsdl1.2-dev xterm libssl-dev libelf-dev -y

3. Clone Yocto Source Repositories

Establish a dedicated workspace directory on your host workstation and mirror the Yocto baseline Poky suite along with the critical structural platform layers targeting the 'kirkstone' release branch:

mkdir ~/rdkb-pi cd ~/rdkb-pi git clone -b kirkstone git://git.yoctoproject.org/poky git clone -b kirkstone https://github.com/openembedded/meta-openembedded.git git clone -b kirkstone https://github.com/agherzan/meta-raspberrypi.git

4. Initialize Build Environment

Source the standard script configuration to instantiate the localized cross-compilation target terminal profile. This populates environment variables and establishes localized shell scopes:

cd ~/rdkb-pi/poky source oe-init-build-env

Execution Note: Running this setup routine automatically shifts your active command-line context into the newly initialized build subdirectory structure:

build/

5. Configure Raspberry Pi 4 Target Machine

Modify the baseline machine flag mapping within the global compilation directive manifest file:

vi conf/local.conf

Append or modify the machine variable to isolate compilation targeting to the Raspberry Pi 4 hardware architecture:

MACHINE = "raspberrypi4"

6. Add Required Meta-Layers to BitBake Engine

Register the cloned out-of-tree hardware abstraction and utility collection recipes directly into your localized BitBake engine layout registry:

bitbake-layers add-layer ../meta-openembedded/meta-oe bitbake-layers add-layer ../meta-openembedded/meta-python bitbake-layers add-layer ../meta-openembedded/meta-networking bitbake-layers add-layer ../meta-raspberrypi

Validate proper inclusion and layered priority hierarchy by executing the evaluation command:

bitbake-layers show-layers

7. Add AP Packages Before Build (Package Customization)

To avoid downstream image deficiencies, explicit networking daemon payloads must be bundled into the target recipe composition block prior to build initiation.

Open your site configuration mapping:

vi conf/local.conf

Append the targeted image package distribution matrix exactly as shown below:

IMAGE_INSTALL:append = " \ hostapd \ dnsmasq \ iw \ wireless-regdb \ "

CRITICAL WARNING / DEPENDENCY NOTE:

Do NOT append or request the legacy package group 'wireless-tools'. In modern Kirkstone environments, this causes an immediate recipe parsing failure due to lack of an upstream provider:

Nothing RPROVIDES wireless-tools

The legacy tools are completely superseded by the modern, lightweight netlink implementation: 'iw'.

8. Execute the Yocto Image Build Process

Instruct the BitBake scheduler engine to assemble, verify dependencies, fetch resources, cross-compile, and build the baseline reference embedded Linux distribution rootfs image:

bitbake core-image-base

Estimated Compilation Windows:

Between 30 minutes to 3 hours, depending heavily on host workstation resource parameters (CPU core density, RAM ceiling, SSD transaction bandwidth, and local network link download metrics).

9. Verify the Generated Target Image Structure

Navigate directly to the localized output deploy filesystem staging pipeline tracking workspace:

cd build/tmp/deploy/images/raspberrypi4

List output images:

ls

Verify the presence of the required core target flash assets below:

core-image-base-raspberrypi4.wic.bz2 core-image-base-raspberrypi4.wic.bmap

10. Decompressing the Image File

Extract the compressed image archive block layer via the bunzip2 tool utility to prepare the raw block layout for raw hardware storage flashing:

bunzip2 core-image-base-raspberrypi4.wic.bz2

Confirm extraction success:

ls *.wic

Expected standard layout confirmation file:

core-image-base-raspberrypi4.wic

11. Inserting and Detecting the Target SD Card

Connect your target physical micro SD media card reader interface into the Linux workstation host. Query the kernel block subsystem layout map to safely discover the device path pointer identifier:

lsblk

Example targeted media identification printout:

sda 29.7G (Target storage disk boundary identified)

12. Unmounting Local SD Card Partitions

Isolate the disk drive path and guarantee system safety by forcing a target partition unmount sequence before stream writing block layers:

sudo umount /dev/sda1 sudo umount /dev/sda2

13. Flashing the Target Image to SD Media

Execute a raw byte stream write transfer across the disk infrastructure layer using the high-level 'dd' system utility. Double-check device paths to prevent accidental host system disk overwrites:

sudo dd if=core-image-base-raspberrypi4.rootfs.wic \ of=/dev/sda \ bs=4M \ status=progress \ conv=fsync

Wait for completion text indicators inside the target terminal console:

records in records out

14. Safe Hardware Removal

Commit all outstanding dirty cache page buffers sitting within volatile host storage registers down onto the permanent flash memory block array:

sync

You may now safely eject and physically remove the SD card media from the host machine reader.

15. Booting the Raspberry Pi Hardware Target

Insert the flashed micro SD media layout card directly into the physical micro-SD slot located on the underside of your Raspberry Pi 4.

Connect the following standard input/output platform interfaces:

Micro-HDMI Output Port to a display monitor

USB Keyboard interface peripheral

USB-C Power Cable (5V / 3A Power Supply Spec Required)

Apply mains voltage power. Allow initial system setup routines to stabilize. At the prompt, access the target platform terminal using the default admin privilege context:

Login: root

16. Verifying Installed AP Packages

Perform a fast path structural sanity interrogation across the system binaries directory space to confirm structural availability of the core system packages:

which hostapd which dnsmasq which iw

Expected binary path mappings verification output:

/usr/sbin/hostapd /usr/sbin/dnsmasq /usr/sbin/iw

17. Confirming Wireless Interface State

Audit the available physical and soft-mapped netlink core wireless device nodes available to the local operating kernel space:

iw dev

Expected system interface output layout block:

wlan0

If the link mapping fails to register explicitly, fall back to evaluation using standard network state utilities:

ip link

18. Configuring WLAN Interface IP Layer

Bind a fixed static logical IPv4 addressing subnet layout topology over the physical link framework layer to initialize it as a structural default gateway network boundary:

ifconfig wlan0 192.168.50.1 netmask 255.255.255.0 up

Inspect the updated link structure parameter values to verify successful interface manipulation:

ifconfig wlan0

19. Creating hostapd System Configuration

Provision the structural path directory meant to house the wireless authentication and link encryption parameter frameworks:

mkdir -p /etc/hostapd

Generate a new custom system configuration map deployment file:

vi /etc/hostapd/hostapd.conf

Populate the configuration target profile manifest parameters exactly as specified below:

interface=wlan0 driver=nl80211 ssid=RDKB_AP hw_mode=g channel=6 wmm_enabled=1 auth_algs=1 ignore_broadcast_ssid=0 wpa=2 wpa_passphrase=Password123 wpa_key_mgmt=WPA-PSK rsn_pairwise=CCMP

CRITICAL PASSPHRASE LENGTH BOUNDARY CONSTRAINT:

The configuration variable parameter 'wpa_passphrase' must conform strictly to standard IEEE 802.11 boundaries, requiring an explicit length constraint between 8 and 63 ASCII text characters.

20. Creating dnsmasq DHCP Configuration

Move the baseline system config file into a standard archival format file to avoid layer configuration conflicts:

mv /etc/dnsmasq.conf /etc/dnsmasq.conf.bak

Open and generate the new targeted operations system descriptor file:

vi /etc/dnsmasq.conf

Populate with specific local subnet leasing allocation directives:

interface=wlan0 dhcp-range=192.168.50.10,192.168.50.100,255.255.255.0,24h

21. Launching hostapd Daemon Interactively

Execute the access point orchestration service tool interactively to evaluate real-time terminal debug feedback layers:

hostapd /etc/hostapd/hostapd.conf

Expected Operational Success Anchor String:

AP-ENABLED

Note: To avoid service termination, preserve the active running process inside the terminal shell window. To launch safely inside a standard background automation layer, pass the daemonize flag:

hostapd -B /etc/hostapd/hostapd.conf

22. Starting dnsmasq Infrastructure Daemon

Initialize the dynamic IP lease distribution sub-layer daemon by pointing it explicitly at your newly created directive config layout:

dnsmasq -C /etc/dnsmasq.conf

Port Conflict Handling Steps:

If the network layer throws an error highlighting a port routing conflict:

port 53 already in use

Isolate the conflicting process running PID identifier via active process tree mapping inspection:

ps ¦ grep dnsmasq

Forcibly terminate the conflicting blocking PID instance:

kill <pid>

Re-execute the initialization command string line to bind interface operational listeners safely:

dnsmasq -C /etc/dnsmasq.conf

23. Verifying Access Point Runtime State

Re-evaluate the structural parameters of your active soft-mapped wireless driver layout map:

iw dev

Expected Operational Mode Realized State Parameter Verification:

type AP

24. Connecting Client Devices

Use an external client station device (such as a smartphone, notebook computer, or external laboratory analysis client) to poll nearby wireless RF airspaces.

Discoverable Broadcast SSID Identifier Lookup Name:

RDKB_AP

Assigned Client Security Access WPA Passphrase Key:

Password123

25. Monitoring Connected Client Stations

Query the internal mac80211 link association table registry stack to verify operational hardware connection status across all remote station client blocks:

iw dev wlan0 station dump

Successful active tracking verification node output layer template:

Station xx:xx:xx:xx:xx:xx

26. Validating Lease & DHCP Assignment Records

Inspect the active leases storage flat-file records kept by the dnsmasq network service daemon to map active layer-3 logical identity targets down to underlying MAC interfaces:

cat /var/lib/misc/dnsmasq.leases

The output array cleanly parses into three distinct data column profiles:

Target Leased Subnet IPv4 Address Assignment

Hardware Interface Layer Physical MAC Address Pointer

Hostname String Identification Flag

27. Comprehensive Troubleshooting Guide

This compilation organizes operational edge-cases, system errors, and correction routines documented during environment development.

Issue 1: bitbake: command not found

Description: The shell context lacks active environment path mapping pointers to localized execution recipes.

Resolution Logic Step:

source oe-init-build-env

Issue 2: Nothing RPROVIDES wireless-tools

Description: The build target recipe requests obsolete legacy tool integration frameworks not present in the Kirkstone release tree.

Resolution Logic Step:

Remove the 'wireless-tools' string definition completely from local.conf. Utilize the modern 'iw' system utility component tool natively.

Issue 3: hostapd passphrase length error

Description: The assigned configuration passphrase string violates the strict criteria boundaries defined by wireless authentication frameworks.

Resolution Logic Step:

Ensure the 'wpa_passphrase' text property array is strictly restricted to a layout size between 8 and 63 alphanumeric text characters.

Issue 4: Station Associated then immediately Deauthenticated

Description: Root causes point to a structural failure vector inside the environment layer configuration. Common culprits include: invalid WPA parameter blocks, mismatched/clashing RF frequency channel selection indexes, physical firmware-driver incompatibilities, or severe DHCP packet drop bounds.

Resolution Logic Step:

Check hostapd.conf parameter compliance, try forcing alternative channel layouts, or audit system messages using dmesg logs.

Issue 5: dnsmasq: port 53 already in use error

Description: A pre-existing system instance daemon is already running and actively listening on Port 53.

Resolution Logic Step:

Isolate the blocking process using 'ps ¦ grep dnsmasq', terminate the active process task using 'kill <pid>', and safely re-launch the targeted manual command tool configuration script.

Issue 6: Access Point connects successfully but WAN/Internet is unavailable

Description: The access point provides operational layer-2 connectivity and localized layer-3 gateway routing. It lacks global Internet visibility because Network Address Translation (NAT) and packet forwarding logic remain unmapped across internal routing tables.

Resolution Logic Step:

Configure standard Linux iptables forwarding rules, enable ip_forward flags inside sysctl configuration properties, and bridge external upstream interfaces (e.g., eth0).

Final Operational Result Topography

Once all steps are executed sequentially, the hardware operates within the following logical topology framework:

Raspberry Pi 4 Target Hardware Platform └── RDKB_AP (Active Layer-2 Broadcast Interface) └── Client Node (Phone/Laptop Connection Bound) ├── Dynamic DHCP Lease via dnsmasq Infrastructure └── WPA2 Link Encryption Security Handled via hostapd